# Caspian Environment Programme Information System

# Monitoring Equipment Specification

Contents

1	Introduction	1

2	General laboratory and field equipment	2

3	General office equipment	3

4	Marine coastal and freshwater habitat monitoring	3

4.3	Monitoring aims and objectives	3

4.4	Monitoring equipment requirements	4

5	Coastal habitat and wetland monitoring	6

5.3	Monitoring aims and objectives	6

# Introduction

This document presents a draft equipment specification for the environmental and ecological monitoring of sensitive sites in the Caspian littoral area.

Monitoring protocols are in development for marine coastal, terrestrial, and wetland habitats.

# General laboratory and field equipment

All participating institutions will require some basic equipment which will service a number of requirements.

Basic laboratory and field equipment

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Oven and associated crucibles and glassware | For drying sediment and biotic samples to measure dry mass | Should be able to maintain stable temperatures up to 100C |
| Muffle furnace | For combusting organic matter and measuring the organic-free weight of sediment and biotic samples | Should be able to maintain stable temperature of at least 650C |
| Balances | Weighing sediment and biotic samples. Preparation of reagents and standards for chemical analysis | 3-place balance (1mg precision) 5-place balance (10 ug precision) |
| Digital and video cameras | Recording site details and sampling events | Not critical, but digital camera should have minimum 3 MP resolution |
| Voice recorders | Recording verbal notes during field work | May be separate item, or can purchase digital camera with voice recording facility |
| Handheld GPS | Position fixing | Essential for precisely locating sampling locations, transects etc during fieldwork |
| Protective clothing | Hard hats, coveralls, gloves, eye protection are required to ensure that all work can be done without putting workers at risk | Need to agree on a basic set of international HSE standards which will be applied to all laboratory and field work |

Analysis of water samples

| Equipment | Parameter | Specification |
| --- | --- | --- |
| UV/Vis spectrophotometer | Nutrient analysis |  |
| Fluorescence spectrophotometer | Chlorophyll and PAH analysis |  |
| Reagents and glassware | For nutrient, chlorophyll and PAH analysis | Should meet requirements of standard international oceanographic and limnological methods |
| Bench water quality meters | pH, dissolved oxygen, conductivity | Required for calibration of field instruments |
| Laboratory water standards | International standard samples for all parameters | Required for calibration and verification of analyses |

Analysis of soil and sediment samples

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Sieves and shaker | Soil and sediment particle size analysis | Standard international sieve series |
| Reagents and glassware | De-flocculation of soils and silt-clay analysis | For standard pipette analysis of fine fractions |
| Hydrometer | Silt-clay analysis | For rapid analysis of fine fractions |

# General office equipment

All participating institutions will require a minimum provision of office equipment.

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Computers & printers | Data processing and document production | Should be minimum 2 GHz, with 512 Mb memory, DVD re-writer |
| WP software | Document production | Should agree on standard software to optimise document exchange and collaboration on document production |
| Statistical software | Data processing and analysis | Should use internationally-established package such as Minitab or SPSS to ensure compatibility of results and conclusions |
| GIS software | For processing of satellite images and creation of associated data layers | Need to ensure that this conforms to an agreed CEP standard – ie that all CEP parties use the same software |
| Reliable internet connection |  |  |
| Office furniture as required |  |  |
| Consumables |  |  |

# Marine coastal and freshwater habitat monitoring

## Monitoring aims and objectives

Marine coastal monitoring will focus on the nearshore environment, in water depths of up to 20m.  It may also be necessary to monitor key freshwater habitats (lakes and rivers) close to the littoral zone. The aim of monitoring will be to secure regular and accurate information on:

- Basic water column chemistry: temperature, dissolved oxygen, salinity, major nutrients (N, P compounds)

- Zoo- and phytoplankton population composition

- Benthic sediment type and structure

- Benthic epifaunal and infaunal community structure and composition

- Aquatic flora (algae and seagrass) community coverage

- Hard (stony or rocky) substrate communities

- Local (ie, resident) fish populations

Benthic monitoring will have the highest priority, since benthic habitats and communities provide an important food source for fish and tend to integrate environmental stresses over long periods of time.

## Monitoring equipment requirements

Field sampling

| Equipment | Purpose | Specification |
| --- | --- | --- |
| 7-m RIB or workboat 2x 50 hp outboard motors Trailer | To provide a platform for nearshore water and benthic sampling | Workboat should provide a safe and stable platform for nearshore sampling. Two engines required for safety reasons. Must be easily transported by trailer for access to coastal and inland water sites |
| Van Veen grab, winch and wire | For sampling soft bottom marine and freshwater habitats | Two grabs and spare wire required. Grab must be light enough to deploy by hand in shallow water |
| Sediment corer | For taking depth cores in soft sediments | 10 cm diameter, acrylic plastic or aluminium, with close-fitting caps. Should be designed to attach to pole for sampling in shallow water |
| Lifejackets, safety harnesses and safety cable | To ensure that sampling can be conducted without risk to participants | Equipment should be to recognised international standards |
| Plankton sampling nets | Paired nets (coarse and fine mesh) with wire, winch and flowmeter | Should be possible to deploy nets by hand or by winch, and nets should be small enough to safely deploy and retrieve from small vessel |
| Sample containers | To receive benthic sediment and plankton samples | HDPE or similar – should be of sufficient capacity to hold anticipated sample volumes, so size will depend on the sampling volumes of the grab and plankton nets |
| Drop-down video | Monitoring of hard substrate and seagrass | Video camera on pole or frame. Waterproof housing (30m). Repeater screen for monitoring from surface during recording |
| ‘Fish finder’ | Acoustic recording device for approximate abundance and size distribution of nearshore fish populations | Propose the use of standard recreational fish finder equipment – basic handheld instrument with drop-down transponder. Can also be used to record water depth at sampling locations |

In situ water quality measurement

| Equipment | Parameters | Specification |
| --- | --- | --- |
| CTD instrument | Depth, pressure, light | 0-100m |
| Horiba U10 or similar | Conductivity Salinity Temperature pH Turbidity | Salinity 0-40 ppt Temperature 0-40 C Turbidity requirements will need to be assessed from existing data |

Water sampling

| Equipment | Parameter | Specification |
| --- | --- | --- |
| NIOS or similar closing bottle Sampling wire and reel Closing device | Collection of water for nutrient and phytoplankton analysis | 2-5 litres capacity |
| Sample reception bottles | As above | 1 and 2-litre glass and food-grade HDPE bottles for sample transportation and preservation |

Sample processing (onshore)

When samples are collected using a small vessel, there will not be sufficient space onboard to carry out preliminary sample processing (for instance, sieving of benthic samples to separate sediment from biota).

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Sediment sample sieve and chute | To separate biota from sediment | Require 0.5 and 1 mm sieves, and wooden or plastic chute over which to conduct sieving |
| Preservatives | To ensure that biota are correctly preserved for further study and storage | Benthos – formalin Plankton – Lugol’s iodine - formalin, phenoxetol |
| Storage containers | Robust containers for long-term archiving of sample material | Should be physically and chemically robust, to ensure that sample material is retained in good condition for future reference |

Analysis of benthic and planktonic biota

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Dissecting microscope | Taxonomic identification of marine invertebrates and flora | 100-200x magnification. Phase contrast capability preferred |
| Compound microscope | For more detailed taxonomy | Up to 1000x magnification, but minimum of 400x. Oil immersion capability required |
| Fibre optic illumination | For use with dissecting microscope | Light source mounted on flexible arm to permit optimal illumination of samples |
| Specimen vials, trays, sample splitters and Petri dishes | Storage, sorting and examination of specimens | Preferably sealable glass for storage, and polystyrene for examination. Sorting trays should be constructed of white plastic |
| Waterproof paper, labels and markers | Secure and reliable marking of sample containers | Must provide indelible long-term labelling to ensure that samples and specimens can be used to maximum benefit over a period of decades |

# Coastal habitat and wetland monitoring

## Monitoring aims and objectives

Coastal and wetland monitoring will focus in the first instance on littoral habitats within 10km of the coast.  Monitoring can be extended further inland once the programme for key littoral habitats is established and is successfully operating.  The objectives of monitoring will be to secure regular information on:

- Soil conditions, including salinisation and erosion

- Basic wetland water quality

- Plant community structure and biomass

- Animal population structure and abundance

- Local bird community structure and abundance

- Grazing pressure from local land use

- Pressure and encroachment from municipal and industrial sources

General equipment

| Equipment | Parameter | Specification |
| --- | --- | --- |
| Sample quadrats | Quantitative determination of plant community composition, abundance and biomass | Standard 1m2 quadrats constructed from durable plastic or aluminium |
| Transect tape or rope | For establishing standard linear transect and defining regular transect observation points | Polypropylene or nylon rope or tape, orange or yellow for maximum visibility |
| Tape measure | For accurately measuring positions and distance | Minimum 25m durable reel-type tape measure |
| Soil corer and auger | For taking soil cores | 5-10 cm diameter, should be capable of taking cores to 50cm in suitable soils |
| Theodolite or laser levelling device | For establishing land and beach slope and profile | Standard surveying instrument. Must be weather- and shock-proof for field work |
| Marker poles | For use in profiling | Aluminium poles marked in alternate red and white at 5cm intervals |
| Binoculars | Bird monitoring | Minimum 20x, preferably with zoom facility |
| Pond nets | Wetland amphibia and insect sampling |  |
| Insect nets | Insect sampling in vegetation |  |

Specialised equipment

For selected sites, it may be appropriate to monitor bird populations in detail.  This may involve the temporary capture of birds for:

- Health, size and weight recording

- Ringing for mark- and re-capture studies to obtain more accurate estimates of population size and movements

Such studies are highly specialised, and require specifically trained and experienced personnel.  They should not be undertaken as part of routine monitoring unless such personnel are available, and unless the need for such monitoring has been fully demonstrated and agreed.  An example of the specialised equipment required for this type of study would be mist netting, which is used to capture birds without damaging them.