Final analysis page for the rapid assessment of Point Source Pollution

Here are the industries you have chosen in your catchment area:

| Name: |  | Catchment: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| POINT SOURCE | Units | BOD kg/yr | TSS kg/yr | Total N kg/yr | Total P kg/yr | Other Pollutants kg/yr |
| Canning and preserving of fruits and vegetables - Tomato processing (products) - 311326 | 1200 - tn raw material | 1560 | 3240 | 0 | 0 |  |
| Canning, preserving and processing of fish, crustacean and similar foods - Fish meal (without solubles) - 311411 | 120 - tn of product | 7464 | 4176 | 0 | 0 | Oil: 2736 |
| Slaughtering, preparing and preserving meat - Poultry processing with blood recovered - 311107 | 120 - 1000 birds | 1428 | 1524 | 0 | 0 | Oil: 672 |
| Petroleum refineries - Cracking refinery - 353002 | 30 - 1000m3 crude | 2187 | 546 | 849 | 0 | Oil: 936 Cr: 7 Phenol: 120 Sulphide: 28 |
| Petroleum refineries - Lub oil refinery - 353004 | 3223 - 1000m3 crude | 699391 | 230444 | 77674 | 0 | Oil: 386760 Cr: 148 Phenol: 26750 Sulphide: 45 |
| Manufacture of soap and cleaning preparations - detergents (chlorosulfonic acid sulfation) - 352314 | 35000 - tn anhydr soap | 105000 | 10500 | 0 | 0 | Oil: 17500 |
| Manufacture of rubber products not elsewhere classified - 355901 | 1919 - no. employees | 28 | 129689 | 0 | 0 |  |
| Nonferrous metal basic industries - Primary aluminum production (aluminium from alumina (anode pre-baking)) - 372003 | 16621 - tn of product | 0 | 83105 | 0 | 0 |  |
| Manufacturing industries not elsewhere classified - 390901 | 1400 - no employees | 2 | 570922 | 0 | 0 |  |
| Sewage - No Treatment - 401101 | 1250000 - per person | 22625000 | 49000000 | 4125000 | 1162500 | Bacteria: 6.25e+021 |
| Sewage - Tertiary Treatment (for bacteria) - 401104 | 1480000 - per person | 5357600 | 11603200 | 3670400 | 695600 | Bacteria: 7.4e+018 |
| TOTAL | - | 28799660 | 61637346 | 7873923 | 1858100 | Oil: 408604 Cr: 155 Phenol: 26870 Sulphide: 73 |