- Ministry of Jihad Agriculture

- Iranian Fisheries Research Organization

- Research Center for Fish Farming in Inland Waters

- The Caspian Sea Ecological Research Center

- with the cooperation of

- Caspian Environment Programme (CEP)

- A Study on the

- Harmful Algal Bloom

in the Southwestern Basin of the Caspian Sea

- Autumn 2006

- Table of Content

Introduction								3

Objectives									4

Project Time Schedule 						5

Technical Performance 						5

Discussion & Results							6

Water Temperature Regime					6

Salinity									7

Chlorophyll A								7

Nutrients									7

Phytoplankton								8

Hetrocapsa Algal Bloom						9

Photo of Hetrocapsa 							10

Phytoplankton Tables 						11

Physical & Chemical Tables 					13

- A Study on the Harmful Algal Bloom

at the Southwestern Section of the Caspian Sea

- Introduction

The Caspian Sea, the largest landlocked body of water in the world with an area of 386,400 square kilometers and 7,250 kilometers of coastline is under severe environmental threats including the water level fluctuations; over fishing; poaching; invasive species; industrial, agricultural and urban pollution; and unsustainable development. The southern Caspian water basin with an area of 17,700 square kilometers (7% of the total Caspian water basin) provides about 5% of incoming waters to the Caspian Sea. The rapid growth of population in the Caspian water basin is one of the factors threatening its aquatic biota.

In early September 2005, an unprecedented algal bloom in the western, central and Iranian coastlines of the Caspian Sea was observed. This phenomenon was filmed and reported at that time. These reports identified the bloom as Cyanophyta, which was never seen in the Caspian waters before. The Iranian Fisheries Research Organization determined that the algae type was Nodularia and it was reflected in its website and national newspapers.

According to the reports of Iranian Fisheries Research Organization, the algal bloom was first seen at northwestern waters of Gilan Province and then it moved towards Anzali and Nowshahr Ports.

The cause of this giant algal bloom is not yet known, however for the formation of Cyanophyta algal bloom, a series of intricate factors are to be present in the water including pollutants, considerable amount of nutrients and appropriate temperature.

The observed algal bloom was quite large and covered an area of 150 square kilometers from Anzali to Nowshahr Ports. The calm sea conditions and appropriate water temperature (25 degrees Centigrade) contributed to its formation.

The reports prepared by the various research centers of the Iranian Fisheries Research Organization have emphasized on the identification of the bloom type and its impact on the Caspian ecosystem. These reports have foreseen the possibility of its reappearance. Although physical and chemical analysis of water and the bloom was conducted, the causes of its formation were not studied.

There has never been a documented report on the detection of harmful algal blooms in the Iranian waters of the Caspian Sea in the past. Nevertheless, their presence in the Anzali Wetlands was reported in limited studies conducted by college students. Moreover, algal blooms have been detected in the Persian Gulf and the Sea of Oman. Unfortunately, no specific study has been conducted on them either. The possible lack of interest for identification of these blooms might be due to their apparent harmless presence in terms of mortality of aquatics or threat to human health. In Europe and America, the excessive amount of nutrients produced by the industrial, agricultural and urban wastes has contributed to the regular formation of algal blooms. These blooms are constantly monitored in USA, Spain, France, England, Italy and Greece to control their adverse effects on the marine environment.

The occurrence of this phenomenon in the Caspian Sea is important and could indicate an alarming signal on the well being of the Caspian environment. It is possible that the formation of algal blooms in the Caspian Sea is related to other phenomena like mass mortality of Caspian seals and fish species, as well as the sharp decline in sturgeon stocks.

In this study, the status of phytoplankton, nutrients, dissolved oxygen, water temperature, chlorophyll type A and water transparency in four transects from Astara to Kiashahr were studied in order to identify their changes in relation to the algal bloom.

- Objectives

- Study the temporal and geographical changes in phytoplankton and nutrients in relation to the algal bloom

- Identify the type of algae involved in the algal bloom and estimate its amount and distribution pattern

- Identify the toxic materials produced in the algal bloom

- Measure the physical, chemical characteristics of water including temperature, PH, dissolved oxygen, water transparency, nutrients (P,N, Si), chlorophyll type A and salinity.

- Project Time Schedule

- Table #1: Project Time Schedule

| Time Schedule | August | Sept. | Oct. | Nov. | Dec. | Jan. | Feb. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Preparation | * | * | * |  |  |  |  |
| Scientific Cruises #1, #2, #3 |  | * | * |  |  |  |  |
| Scientific Cruises #4, #5 |  |  |  | * |  |  |  |
| Analysis of abiotic factors in water samples |  | * | * | * |  |  |  |
| Analysis of biotic factors in water samples |  | * | * | * | * | * |  |
| Preliminary report |  |  |  | * |  |  |  |
| Analysis of data |  |  |  | * | * |  |  |
| Final report |  |  |  |  |  |  | * |

- Technical Performance

The first scientific cruise was commenced on 19th September 2006 from the Anzali Port and was concluded on 22nd of September 2006 in Lisar. The purpose of this cruise was to take water samples from the depths of 20, 50 and 100 meters.

The second scientific cruise began on 1st October 2006 from Kiashahr and ended on 8th October 2006 in Lisar.

The third scientific cruise was commenced on 4th of November 2006 from Kiashahr and was concluded on 5th of November 2006 in Anzali Port. During this cruise samples were collected. However, the stormy conditions of the sea have prevented the other scientific cruises to be conducted and samples be taken.

Afterwards, the laboratory analysis of the samples was conducted and the complete data on hydro-chemical status of the samples was obtained. Also, the phytoplankton samples from the first and second scientific cruises were analyzed and their biomass was determined. The preliminary data is presented in this report and the comprehensive information and analysis of samples will be incorporated into the final report.

- Discussion & Results

- Water Temperature Regime

Water temperature is one of the main factors in metabolism of the aquatic species. The temperature regime of the Caspian Sea has a significant impact on its water cycle behavior. During the scientific cruises, the water temperature was measured up to the depth of 100 meters in various levels. The results indicate that the temperature difference in various levels of the water is considerable and its trend varies in different regions. In Lisar region, the thermo-cline in depths of 20 to 35 meters is established with the temperature difference of 10 degrees Centigrade. While in Astara, it occurs in depths of 30 to 35 meters with the temperature difference of 6 degrees Centigrade, in Anzali Port, in depths of 40 to 45 meters with the temperature difference of 13 degrees Centigrade and in Kiashahr, in depths of 30 to 40 meters with temperature difference of 13 degrees Centigrade. The presence of thermo-cline and its distance from the water surface is influential in biological activities and the amount of nutrients. Therefore, the biological activities at the depths lower than the thermo-cline level tend to diminish.

- Figure #1: Temperature Changes in Various Regions

Vertical Pattern of Water Temperature in Kiashahr, Astara, Anzali Port and Lisar (Temperature in Centigrade versus Depth of water)

- Salinity

The water salinity in the areas under study (Tables #7 to #9) varies between 9.26 grams per liter to the maximum of 12.67 grams per liter. The lowest amount is registered at the depth of 20 meters near Anzali Port, which apparently is due to the effect of fresh waters in the nearby areas. The maximum salinity was observed at the depth of 100 meters near the Anzali Port and the depth of 50 meters near Lisar region. The salinity levels of the Caspian coastlines are instrumental in the water current trends and are important in identification of hydro-chemical status of the water.

- Chlorophyll A

The concentrations of chlorophyll A (Tables #7 to #9), which is one of the indicators of phytoplankton biomass, are studied in this project. The concentrations of chlorophyll A were measured in three intervals and they varied from the minimum of 2.71 micrograms per liter at the depth of 20 meters in Lisar region to the maximum of 35.25 micrograms per liter at the depth of 20 meters near Anzali Port. These studies indicate that the fresh waters of Sefidroud River and the water basin of the Anzali Wetland play an important role in the biological activities of the southern coastlines of the Caspian Sea. The levels of chlorophyll A registered in this project are much higher than the numbers registered in 1994 (chlorophyll A concentrations of 0.54 to 1.34 micrograms per liter). The sharp increase could indicate serious changes in the environment of the Caspian coastal areas, which needs more in depth studies.

- Nutrients

The concentrations of nitrogen and phosphor nutrients (Tables #7 to #9) at the Caspian coastlines are relatively high. The total nitrogen concentrations vary from the minimum of 2.31 milligrams per liter to the maximum of 2.335 milligrams per liter at the depths of 20 meters near Anzali Port. The concurrent increase in total nitrogen with the reduction in salinity in this station indicates that a large volume of fresh water from the Anzali Wetland is injected into the coastal region. This phenomenon is instrumental in the high concentrations of phosphor (0.5 milligrams per litter) and chlorophyll A (32.25 Micrograms per litter).

The horizontal changes in the total nitrogen concentration in Lisar and Astara reveal that contrary to the Anzali Port and Kiashahr, the total nitrogen concentration intensifies as the distance from the coastline increases. The total nitrogen concentration at the depth of 20 meters from the coast is 0.593 and at the depth of 100 meters is 0.589 milligrams per liter. In the 2nd scientific cruise, the changes in the total nitrogen concentration were amplified as the distance from the coastline increased, as it jumped from 0.282 to 0.463 milligrams per litter at the depth of 20 meters. The same trend was observed for phosphor concentration. Therefore, the factors influential in this trend are to be studied more closely.

The total phosphor concentrations fluctuate from the minimum of 0.026 to the maximum of 0.500 milligrams per liter and the maximum amounts of phosphor are registered at the Anzali Port and Kishahr Port.

The N/P ratio is an indicator for identification of the factors limiting the growth of primary producers. This ratio varies in different planktons, where the maximum ratio is 7 to 1. Therefore, any amount of nitrogen less than 7 could be considered as a limiting factor. The N/P ratio varied from 2/8 to 17/1 in this study. The minimum and maximum amounts were registered in Astara and Lisar region, respectively. Meanwhile, the total mean average was over 7.

- Phytoplankton

Phytoplankton is the primary link in the food chain of the aquatic ecosystems, which are very sensitive to the changes in the environment. In the first scientific cruise, at the southwestern stations of the Caspian Sea (Tables #2 to #6), a total of 34 phytoplankton species from five phylum of Cyanophyta (6 species), Chrysophyta (15 species), Pyrrophyta (7 species), Chlorophyta (4 species) and Euglenophyta (2 species) were identified. In comparison with the data from 1994 (Research Center of Mazandaran Province), which indicated a total of 66 species from the Cyanophyta (9 species), Chrysophyta (33 species), Pyrrophyta (11 species), Chlorophyta (9 species) and Euglenophyta (4 species), the number of species registered are quite different. The Chrysophyta phylum with 15 species was registered with the highest number of phytoplankton species in this scientific cruise and the same supremacy in the number of species was reported in the previous studies (Ali Ganjian, 2003; Research Center of Mazandaran Province, 2004). Therefore, the number of observed species is much lower than before. This is also true for other phytoplankton phylum, where the variety of species has decreased in comparison with the numbers from 2004.

The Nodularia harveyana, a phytoplankton species that bloomed in late summer of 2005 was also reported in the coastal areas of Gilan Province in 2004 (Research Center of Mazandaran Province). But, the phytoplankton species were not identified. In this project, the number of phytoplankton species was different in the four studied regions. Anzali Port with 22 species from the phytoplankton Phylum had the highest biodiversity followed by Kiashahr with 16 species, Lisar with 8 species and Astara with 7 species. The entrance of fresh waters into the Anzali Port region and Kiashahr through the breakwater canals, discharge of Sefidroud River into the sea and the closeness of deep sea sections to the shorelines could be the main reasons for the increase in the number of phytoplankton species in these regions. The Cyclotella menenghiniona with the total number of 89,400,000 and biomass of 894 mg/l had the highest biomass between phytoplankton species. This species belongs to the Chrysophyta phylum.

- Hetrocapsa Algal Bloom

The interesting phenomenon observed on 6th October 2006 was the creation of an algal bloom outside the studied areas. This algal bloom at the Anzali Port area covered the region between the breakwater canal and the Hassanroud Township up to 300 meters away from the shorelines. The sea water at this area had a reddish brown color (Figure #2). Water samples were taken from the sea and it was concluded that the reddish color was due to the creation of an algal bloom called Red Tide. An algae from Heterocapsa genus, Peridinaceae family, Peridiniales order, Dinophceae class and Pyrrophyta phylum was responsible for this algal bloom. This genus is predominantly seen in brackish waters and has a short life span. Also, it is very sensitive to the stabilizing solutions and its cellular walls are quickly dissolved in diluted formalin solutions. This genus can not be detected in samples stabilized by formalin.

It is interesting to point out that the above-mentioned incident occurred after a flood at the Anzali Wetland water basin and the Azolas brought to the coastal areas by the flood were covered by the algal bloom. The phenomenon gradually disappeared in three days.

- Figure #2: Heterocapsa Algal Bloom with Reddish Brown Color (Red Tide) along with Azola

6th October 2006

- Table #2: Phytoplankton Species, Pyrrophyta Phylum in the 4 Studied Areas

| Sample Code | Phylum | Species | Number per Cubic Meter | Biomass mg/m3 | Station | Depth (Meter) |
| --- | --- | --- | --- | --- | --- | --- |
| 64 | Pyrrophyta | Exuviella cordata | 27600000 | 55.2 | Anzali | 20 |
| 81 | Pyrrophyta | Gymnodinium variabile | 1200000 | 2.4 | Anzali | 20 |
| 64 | Pyrrophyta | Exuviella cordata | 3200000 | 6.4 | Anzali | 50 |
| 64 | Pyrrophyta | Exuviella cordata | 800000 | 1.6 | Anzali | 100 |
| 64 | Pyrrophyta | Exuviella cordata | 7000000 | 14 | Kiashahr | 20 |
| 64 | Pyrrophyta | Exuviella cordata | 4000000 | 8 | Kiashahr | 50 |
| 168 | Pyrrophyta | Prorocentrum scutellum | 800000 | 26.4 | Kiashahr | 50 |
| 393 | Pyrrophyta | Peridinium sp | 800000 | 9.6 | Kiashahr | 100 |
| 64 | Pyrrophyta | Exuviella cordata | 1600000 | 3.2 | Kiashahr | 100 |
| 64 | Pyrrophyta | Exuviella cordata | 22400000 | 44.8 | Astara | 20 |
| 64 | Pyrrophyta | Exuviella cordata | 2400000 | 4.8 | Astara | 50 |
| 64 | Pyrrophyta | Exuviella cordata | 1000000 | 2 | Astara | 100 |
| 70 | Pyrrophyta | Glenodinium lenticula | 1000000 | 4 | Astara | 100 |
| 64 | Pyrrophyta | Exuviella cordata | 3000000 | 6 | Lisar | 20 |
| 156 | Pyrrophyta | Peridinium latum | 1000000 | 30 | Lisar | 50 |
| 73 | Pyrrophyta | Goniaulax digitale | 1000000 | 12 | Lisar | 100 |

- Table #3: Phytoplankton Species, Chlorophyta Phylum in the 4 Studied Areas

| Sample Code | Phylum | Species | Number per Cubic Meter | Biomass mg/m3 | Station | Depth (Meter) |
| --- | --- | --- | --- | --- | --- | --- |
| 391 | Chlorophyta | Crucigenia tetrapedia | 2400000 | 0.024 | Anzali | 20 |
| 173 | Chlorophyta | Scenedesmus quadricauda | 2400000 | 1.44 | Anzali | 20 |
| 13 | Chlorophyta | Ankistrodesmus acicularis | 1200000 | 0.64 | Anzali | 20 |
| 392 | Chlorophyta | Sceletonema costatum | 7200000 | 2.16 | Kiashahr | 50 |

- Table #4: Phytoplankton Species, Cyanophyta Phylum in the 4 Studied Areas

| Sample Code | Phylum | Species | Number per Cubic Meter | Biomass mg/m3 | Station | Depth (Meter) |
| --- | --- | --- | --- | --- | --- | --- |
| 152 | Cyanophyta | Oscillatoria sp | 60600000 | 13.69 | Anzali | 20 |
| 151 | Cyanophyta | Oscillatoria limosa | 600000 | 0.135 | Anzali | 20 |
| 11 | Cyanophyta | Anabaenopsis cunningtonii | 13800000 | 33.12 | Anzali | 20 |
| 94 | Cyanophyta | Lyngbya sp | 600000 | 0.24 | Anzali | 20 |
| 180 | Cyanophyta | Spirulina laxissma | 800000 | 1.04 | Anzali | 50 |
| 152 | Cyanophyta | Oscillatoria sp | 4000000 | 0.9 | Anzali | 50 |
| 152 | Cyanophyta | Oscillatoria sp | 800000 | 0.18 | Anzali | 100 |
| 152 | Cyanophyta | Oscillatoria sp | 27000000 | 6.1 | Kiashahr | 20 |
| 152 | Cyanophyta | Oscillatoria sp | 16800000 | 3.79 | Kiashahr | 50 |
| 152 | Cyanophyta | Oscillatoria sp | 4000000 | 0.9 | Astara | 50 |
| 152 | Cyanophyta | Oscillatoria sp | 1000000 | 0.226 | Lisar | 50 |

- Table #5: Phytoplankton Species, Chrysophyta Phylum in the 4 Studied Areas

| Sample Code | Phylum | Species | Number per Cubic Meter | Biomass mg/m3 | Station | Depth (Meter) |
| --- | --- | --- | --- | --- | --- | --- |
| 48 | Chrysophyta | Cyclotella menenghiniana | 89400000 | 894 | Anzali | 20 |
| 83 | Chrysophyta | Gyrosigma attenuatum | 1200000 | 57.6 | Anzali | 20 |
| 99 | Chrysophyta | Melosira varians | 1200000 | 4.8 | Anzali | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 8400000 | 6.37 | Anzali | 20 |
| 187 | Chrysophyta | Synedra ulna | 600000 | 3.6 | Anzali | 20 |
| 28 | Chrysophyta | Chaetoceros sp | 600000 | 0.271 | Anzali | 20 |
| 125 | Chrysophyta | Nitzschia acicularis | 8400000 | 8.4 | Anzali | 20 |
| 136 | Chrysophyta | Nitzschia reversa | 2400000 | 2.4 | Anzali | 20 |
| 140 | Chrysophyta | Nitzschia sp | 600000 | 1.2 | Anzali | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 3200000 | 4.24 | Anzali | 50 |
| 125 | Chrysophyta | Nitzschia acicularis | 800000 | 0.8 | Anzali | 50 |
| 142 | Chrysophyta | Nitzschia tenirustris | 800000 | 1.36 | Anzali | 50 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 4800000 | 4.37 | Anzali | 100 |
| 136 | Chrysophyta | Nitzschia reversa | 800000 | 0.8 | Anzali | 100 |
| 197 | Chrysophyta | Thalassiosira sp | 800000 | 5.3 | Anzali | 100 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 21000000 | 210 | Kiashahr | 20 |
| 392 | Chrysophyta | Sceletonema costatum | 12000000 | 3.6 | Kiashahr | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 13000000 | 17.26 | Kiashahr | 20 |
| 83 | Chrysophyta | Gyrosigma attenuatum | 8000000 | 384 | Kiashahr | 20 |
| 125 | Chrysophyta | Nitzschia acicularis | 47000000 | 47 | Kiashahr | 20 |
| 136 | Chrysophyta | Nitzschia reversa | 1000000 | 1 | Kiashahr | 20 |
| 142 | Chrysophyta | Nitzschia tenirustris | 2000000 | 3.4 | Kiashahr | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 17600000 | 23.37 | Kiashahr | 50 |
| 125 | Chrysophyta | Nitzschia acicularis | 50400000 | 50.4 | Kiashahr | 50 |
| 54 | Chrysophyta | Diatoma volgare | 1600000 | 10.85 | Kiashahr | 50 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 13600000 | 136 | Kiashahr | 50 |
| 83 | Chrysophyta | Gyrosigma attenuatum | 800000 | 38.4 | Kiashahr | 50 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 800000 | 8 | Kiashahr | 100 |
| 392 | Chrysophyta | Sceletonema costatum | 800000 | 0.24 | Kiashahr | 100 |
| 125 | Chrysophyta | Nitzschia acicularis | 11200000 | 11.2 | Kiashahr | 100 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 2400000 | 3.18 | Astara | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 11200000 | 14.87 | Astara | 50 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 16800000 | 168 | Astara | 50 |
| 125 | Chrysophyta | Nitzschia acicularis | 1600000 | 1.6 | Astara | 50 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 11000000 | 14.6 | Astara | 100 |
| 125 | Chrysophyta | Nitzschia acicularis | 1000000 | 1 | Astara | 100 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 5000000 | 6.64 | Lisar | 20 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 9000000 | 90 | Lisar | 20 |
| 191 | Chrysophyta | Thalassionema nitzschiodes | 7000000 | 9.29 | Lisar | 50 |
| 48 | Chrysophyta | Cyclotella menenghiniana | 12000000 | 120 | Lisar | 50 |
| 125 | Chrysophyta | Nitzschia acicularis | 3000000 | 3 | Lisar | 50 |

- Table #6: Phytoplankton Species, Euglenophyta Phylum in the 4 Studied Areas

| Sample Code | Phylum | Species | Number per Cubic Meter | Biomass mg/m3 | Station | Depth (Meter) |
| --- | --- | --- | --- | --- | --- | --- |
| 63 | Euglenophyta | Euglena viridis | 1800000 | 14.4 | Anzali | 20 |
| 197 | Euglenophyta | Trachelomonas sp | 800000 | 5.3 | Astara | 20 |
| 197 | Euglenophyta | Trachelomonas sp | 3000000 | 19.9 | Lisar | 50 |

- Table #7: Results of the 1st Scientific Cruise, Analysis of Physical and Chemical Parameters

of Sea Water (Algal Bloom Project)

|  |  |  |  | mg/l |  |  |  |  | Astara |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.001 | 0.03 | 0.007 | 0.609 | 0.018 | 0.051 | 0.09 | 12.62 | 3.68 |
| 50 meters | 0.006 | 0.029 | 0.005 | 0.546 | 0.017 | 0.056 | 0.09 | 12.62 | 4.86 |
| 100 meters | 0.0007 | 0.032 | 0.003 | 0.507 | 0.033 | 0.054 | 0.1 | 12.62 | 3.41 |
|  |  |  |  | mg/l |  |  |  |  | Lisar |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l)) |
| 20 meters | 0.001 | 0.028 | 0.004 | 0.589 | 0.02 | 0.051 | 0.081 | 12.62 | 2.71 |
| 50 meters | 0.0009 | 0.016 | 0.005 | 0.542 | 0.022 | 0.048 | 0.084 | 12.67 | 4.07 |
| 100 meters | 0.0009 | 0.022 | 0.004 | 0.593 | 0.023 | 0.074 | 0.09 | 12.63 | 2.91 |
|  |  |  |  | mg/l |  |  |  |  | Anzali Port |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.002 | 0.036 | 0.011 | 2.335 | 0.036 | 0.5 | 0.387 | 9.26 | 35.25 |
| 50 meters | 0.001 | 0.028 | 0.009 | 0.707 | 0.031 | 0.215 | 0.115 | 12.35 | 7.92 |
| 100 meters | 0.0007 | 0.025 | 0.008 | 0.637 | 0.027 | 0.062 | 0.273 | 12.66 | 8.69 |
|  |  |  |  | mg/l |  |  |  |  | Kiashahr |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.0007 | 0.013 | 0.006 | 0.687 | 0.004 | 0.071 | 0.22 | 12.43 | 17.39 |
| 50 meters | 0.0007 | 0.014 | 0.005 | 0.815 | 0.004 | 0.088 | 0.21 | 12.47 | 12.47 |
| 100 meters | 0.001 | 0.027 | 0.003 | 0.596 | 0.005 | 0.063 | 0.27 | 12.45 | 8.89 |

- Table #8: Results of the 2nd Scientific Cruise, Analysis of Physical and Chemical Parameters

of Sea Water (Algal Bloom Project)

|  |  |  |  | mg/l |  |  |  |  | Astara |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | n.d | 0.005 | 0.009 | 0.276 | 0.038 | 0.056 | 0.126 | 12.41 | 5.46 |
| 50 meters | n.d | 0.007 | 0.008 | 0.231 | 0.018 | 0.082 | 0.098 | 12.58 | 9.69 |
| 100 meters | n.d | 0.009 | 0.006 | 0.286 | 0.013 | 0.069 | 0.095 | 12.62 | 7.55 |
|  |  |  |  | mg/l |  |  |  |  | Lisar |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | n.d | 0.009 | 0.007 | 0.282 | 0.01 | 0.026 | 0.131 | 12.44 | 6.4 |
| 50 meters | 0.001 | 0.015 | 0.005 | 0.501 | 0.007 | 0.032 | 0.088 | 12.57 | 4.85 |
| 100 meters | n.d | 0.006 | 0.007 | 0.463 | 0.005 | 0.027 | 0.079 | 12.58 | 6.8 |
|  |  |  |  | mg/l |  |  |  |  | Anzali Port |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.002 | 0.009 | 0.018 | 0.57 | 0.019 |  | 0.29 | 12.45 | 10.14 |
| 50 meters | 0.001 | 0.006 | 0.013 | 0.554 | 0.028 |  | 0.101 | 12.53 | 4.64 |
| 100 meters | n.d | 0.005 | 0.008 | 0.525 | 0.021 |  | 0.042 | 12.58 | 3.3 |
|  |  |  |  | mg/l |  |  |  |  | Kiashahr |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.002 | 0.007 | 0.008 | 0.669 | 0.018 |  | 0.161 | 12.35 | 5.7 |
| 50 meters | n.d | 0.01 | 0.005 | 0.489 | 0.016 |  | 0.139 | 12.37 | 5.6 |
| 100 meters | n.d | 0.009 | 0.003 | 0.647 | 0.024 |  | 0.013 | 12.38 | 6.1 |

- Table #9: Results of the 2nd Scientific Cruise, Analysis of Physical and Chemical Parameters

of Sea Water (Algal Bloom Project)

|  |  |  |  | mg/l |  |  |  |  | Anzali Port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.001 | 0.013 | 0.023 | 0.49 | 0.03 | 0.062 | 0.474 | 11.52 | 18.27 |
| 50 meters | 0.002 | 0.009 | 0.02 | 0.542 | 0.036 | 0.051 | 0.035 | 12.46 | 5.54 |
| 100 meters | 0.003 | 0.011 | 0.013 | 0.493 | 0.04 | 0.064 | 0.05 | 12.67 | 6.66 |
|  |  |  |  | mg/l |  |  |  |  | Kiashahr |
| Station | NO2 | NO3 | NH4 | T.N | PO4 | T.P | SiO2 | Salinity (PPT ) | Chlorophyll A (ug/l) |
| 20 meters | 0.002 | 0.004 | 0.009 | 0.549 | 0.018 | 0.061 | 0.183 | 12.18 | 15.75 |
| 50 meters | 0.002 | 0.004 | 0.007 | 0.496 | 0.026 | 0.053 | 0.13 | 12.33 | 8.98 |
| 100 meters | 0.002 | 0.005 | 0.007 | 0.648 | 0.029 | 0.051 | 0.124 | 12.34 | 9.88 |